KwaZulu-Natal Department of Transport

Information about our GIS digital data:

With regards to the digital data CD that you obtained from this office, our Provincial road network consists of Provincial Main roads (class P) which needs to be displayed in red, Provincial District roads (class D) which needs to be displayed in dark green and Provincial Local roads (class L) which needs to be displayed in dark blue (navy blue). Our data sets also contain National roads (class N) which need to be displayed in a lighter blue (similar to royal blue). Therefore when you use any of our Provincial Road Network shapefiles, you must only display class of roads N, P, D and L. Our Department does not govern the National roads, but you can display / use this data.

Our SurfaceEvents shapefile includes the type of road surface, e.g. blacktop, gravel or concrete and for RISFSA roads there is a type Earth road as well. A Provincial Main road with a surface type Blacktop needs to be displayed as a solid red line, Provincial Main road of surface type Gravel needs to be displayed as a red line with black stripes at regular intervals and our Provincial Main roads with surface type Concrete needs to be displayed as a red line with grey stripes at regular intervals.  Similarly the 3 different surface types for District and Local roads will be displayed in the same manner except that the red will be replaced by green for District roads and navy blue for Local roads i.e. a Provincial District road with a surface type Blacktop needs to be displayed as a solid green line, Provincial District road of surface type Gravel needs to be displayed as a green line with black stripes at regular intervals and our Provincial District roads with surface type Concrete needs to be displayed as a green line with grey stripes at regular intervals.  Please refer to the accompanying map called “Surface Example”.

Our Road_Network_measured shapefile has been dynamically segmented, whereas our Road_Network_unmeasured shapefile is basically just the road centre line and as per above, our SurfaceEvents shapefile consists of our measured network together with the surface type information. Any of these 3 shapefiles can be used to display our Provincial Road Network but to preferably use the measured network which will always be distributed when requesting our data.

Towards the latter half of 2011 the KwaZulu-Natal Department of Transport, in conjuction with SSI Civil Engineering Consultants and MHP Geospace, completed a project whereby each and every road down to a level of non-motorised tracks were identified on imagery and digitized accordingly.  All of these roads then underwent a process of classifying each road by the new RISFSA road classification system that was introduced as a new National road classification system.  The Road Infrastructure Strategic Framework for South Africa (RISFSA) that has been adopted at a National level is the blueprint for the planning and development of the road infrastructure. RISFSA details a functional road classification system that is to be adopted and implemented by all roads authorities in the country.  The KwaZulu-Natal Department of Transport (KZN - DOT) have amalgamated the KZN RISFSA Road network with their Provincial Road network to form one combined road network for the whole Province.  However this does not mean that KZN-DOT are responsible for this whole network or maintains this whole network – KZN-DOT are still only responsible for Provincial Main (P), Provincial District (D) and Provincial Local (L) roads and the rest of the RISFSA network will belong to the various respective Municipalies, Private entities, etc.

During 2014 the KwaZulu-Natal Department of Transport (KZN-DOT) adopted a new Road Classification System called the South African Road Classification and Access Management System (RCAM) for which the Committee of Transport Officials (COTO) braught out the TRH26 Manual in this regard in August 2012.  This RCAM data has been incorporarted into our Road Surface event theme and is part of the Provincial Roads Surface layer called “ProvRds_Surface_RCAM_date” (used to be called SurfaceEvents).  KZN-DOT is still only responsible for class P, D or L roads and their respective Ramps (some of the class G roads).