CONVERSION FROM RISFSA TO RCAM ROAD CLASSIFICATION CLASSES

24 April 2012

With the introduction of the South African Road Classification and Access Management Manual, significant changes have been made to the functional classification descriptions in comparison to those contained in the 2006 Road Infrastructure Strategic Framework for South Africa.

Functional classifications previously assigned to roads within KwaZulu-Natal now require modification to align with the RCAM descriptions. The proposed conversion methodology is described below:

ROAD CLASSIFICATION PROCEDURE

1.	Mark up Urban Boundaries

Urban areas include the following:

- Clusters of erven provided for residential or commercial purposes. Agricultural farms and small-holdings are to be excluded;

- Significant clusters of households or commercial buildings in both formal and informal areas. The spacing between residences is typically less than 100 metres. The Eskom polygons can be used to assist in the identification of these areas;

- Identified industrial and commercial areas; and

- Areas within the former TLC boundaries

Developments planned for completion within 5 years should be included when marking up urban and rural area boundaries.

- Review municipal IDP’s, SDF’s, etc

- Check municipal zoning

- Consult with the municipalities

Base the boundaries between urban and rural areas on land use determined from imagery, municipal IDP’s, SDF’s and other planning documents. Where feasible, use cadastral boundaries as the boundary between urban and rural areas. Natural features such as rivers can also be used for the boundaries.

Class 1, Class 2, and Class 3 roads located on the boundary of an urban area are to be excluded from the urban area. Class 4 and Class 5 roads on the boundary of an urban area are to be included within the urban area.

2.	Sub-divide urban areas into residential areas, commercial areas and residential commonage and commercial commonage

Small shopping centres and isolated businesses in residential areas should still be included as part of a residential area.

Commonage includes open areas within urban boundaries where:

- Future urban type development is anticipated

- A park or reserve under the control of the municipality is anticipated.

Commonage areas should be treated as residential commonage, unless specifically marked as commercial commonage.

3.	Plan consolidation of urban areas in consultation with the municipalities

The goal should be to consolidate and infill urban areas to prevent peri-urban sprawl. Separate urban areas in close proximity to each other should be linked together.

4.	Convert the existing RISFSA Classes to initial RCAM Classes according to the attached schedule.

This will be carried out as an automated process

5.	Review and adjust the classifications resulting from the automated conversion.

6.	Classify roads in accordance with the RCAM procedure and manual

7.	Check the relative percentages between classes and adjust if percentages differ substantially from the RCAM guidelines

On completion of the initial classification and adjustment process, an iterative process may be followed to bring the relative percentages between classes closer to the RCAM guideline figures.

CLASSIFICATION CONVERSIONS SCHEDULE

The following schedule indicates the process to be used to convert from the previous RISFSA classifications to the new RCAM functional classifications. Once the automated conversion process has been completed, manual adjustments will be necessary.

Note: The RCAM Classes will be split between 3 separate fields, as indicated by the dotted lines in the schedule below. This is to facilitate easier management of the database.

| RISFSA CLASS | RCAM CLASS |  |  | COMMENTS |
| --- | --- | --- | --- | --- |
| 1 | R U | 1 1 |  | Rural Principal Arterials – R1 Convert all Class 1 roads to Class R1. Urban Principal Arterials – U1 Class U1 roads and adjustments to R1 roads will be identified and captured by SSI. Note: The classification descriptions and methodology differ between R1 roads and U1 roads. Some R1 roads will be continuous as R1 roads through rural and urban areas. In metropolitan areas and large Local Municipalities, some Class 2 roads may need to be changed to Class U1 roads during the conversion process. |
| 2 | R U | 2 2 |  | Rural Major Arterials – R2 Convert all Class 2 roads to Class R2. Urban Major Arterials – U2 Class U2 roads and adjustments to R2 roads will be identified and captured by SSI. Note: The classification descriptions and methodology differ between R2 roads and U2 roads. Some R2 roads will be continuous as R2 roads through rural and urban areas. In metropolitan areas and large Local Municipalities, some Class 3 roads may need to be changed to Class U2 roads during the conversion process. |
| 3 | R U | 3 3 |  | Rural Minor Arterials – R3 Convert all Class 3 roads to Class R3. Urban Minor Arterials – U3 Class U3 roads and adjustments to R3 roads will be identified and captured by SSI. Note: The number of Class 3 roads may increase. The classification descriptions and methodology differ between R3 roads and U3 roads. Some R3 roads will be continuous as R3 roads through rural and urban areas. In metropolitan areas and large Local Municipalities, some Class 4 roads may need to be changed to Class U3 roads during the conversion process. |
| 4A & 4B | U U R | 4 4 4 | a b | Commercial Collector Street – U4a Convert 4A and 4B roads in Urban Commercial areas and Commercial Commonage areas to U4a. Residential Collector Street – U4b Convert 4A and 4B roads in Urban Residential areas and Residential Commonage areas to U4b. Rural Collector Roads-R4 Convert 4A and 4B roads in Rural areas to R4. Adjustments to these will then be captured by SSI. Note: Several Class 4 roads may change to Class 3 roads. |
| 5A & 5B | U U R | 5 5 5 | a b | Commercial Collector Street – U5a Convert 5A and 5B roads in Urban Commercial areas and Commercial Commonage areas to U5a. Residential Collector Street – U5b Convert 5A and 5B roads in Urban Residential areas and Residential Commonage areas to U5b. Rural Collector Roads-R5 Convert 5A and 5B roads in Rural areas to R5. Adjustments to these will then be captured by SSI. |
| 6A | U U | 6 6 | a b | Urban Walkways – U6 Pedestrian Priority Street or Area – U6a Convert 6A roads and walkways to U6a Pedestrian Only Street or Area – U6b These will be identified and captured by SSI. Note: Parking areas, delivery vehicle areas, public transport termini, equestrian paths, etc. will need to be identified and added. |
| 6B | R | 6 |  | Rural Walkways – R6 Convert 6B to R6 Adjustments to these will be done by SSI. |
| 7 |  | 7 |  | Retain existing Class 7 classifications. |